Budapesti Műszaki és Gazdaságtudományi Egyetem

Villamosmérnöki és Informatikai Kar

MIT Tanszék

Kovács Balázs

FPGA alapú rendszerek fejlesztése Házi feladat

BUDAPEST, 2026

Tartalomjegyzék

Tartalomjegyzék	2

1.	Feladat specifikációja	3

2.	Hardver leírás	4

2.1.	Modul szintű blokkvázlat	4

2.2.	Modulok részletes ismertetése	5

2.2.1.	RGB2Y modul	5

2.2.2.	BRAM sorbuffer-lánc	8

2.2.3.	2D 5x5 FIR szűrő	15

2.3.	Processzoros alrendszer	25

2.3.1.	Blokkvázlat	25

2.3.2.	Címkiosztás	26

2.3.3.	Implementációs eredmények	26

2.4.	Teljes rendszer (top_module)	27

3.	Szoftver leírás	33

3.1.	A MicroBlaze szoftvere	33

3.1.1.	Folyamatábra	34

3.1.2.	Megvalósított függvények	35

3.1.3.	Kód	35

3.1.4.	Tesztelés	40

Feladat specifikációja

A feladat célja egy olyan FPGA-alapú képfeldolgozó rendszer megvalósítása volt, amely valós időben képes egy HDMI bemeneten érkező videójelet feldolgozni, majd a módosított képet HDMI kimeneten továbbítani. A rendszer központi funkciója egy 5×5 méretű FIR szűrő implementálása, amelynek együtthatói futás közben is módosíthatók egy MicroBlaze alapú processzoros alrendszeren keresztül.

A tervezett rendszer működése során a HDMI bemenetből érkező RGB képadatokat először szürkeárnyalatos jellé kell alakítani. Ezt követően a képpontfolyam egy több soros bufferen halad át, amely biztosítja, hogy az aktuális pixel környezetéből előálljon a 5×5-ös mintavételi ablak. Az így létrejövő pixelkörnyezetet dolgozza fel a FIR szűrő, amely a beállított együtthatók alapján állítja elő a kimeneti pixel értékét.

A képfeldolgozás folyamatos adatáramlás mellett kell történjen, miatt különös figyelmet kellett fordítani a pufferelésre, a jelfeldolgozó pipeline késleltetéseinek kezelésére, valamint a szinkronjelek megfelelő igazítására.

Hardver leírás

A hardver két nagyobb logikai részre bontható. Az első rész a képfeldolgozó adatút, amely a HDMI vevőt, a szürkeárnyalatos átalakítót, a BRAM alapú vonalpuffert, az 5×5 FIR szűrőt és a HDMI adót tartalmazza. A második rész a MicroBlaze alapú vezérlőrendszer, amely UART kapcsolaton keresztül fogadja a felhasználói bemenetet, majd AXI Lite buszon keresztül beírja a szűrőregisztereket, amihez segítségül az együttható bank szolgál, mely egy AXI periféria modul.

Modul szintű blokkvázlat

Mivel a  HDMI RX és TX oldalakat, illetve a Clock Gen blokkot egy kiinduló projektben megkaptuk, így ezek részletezésére ezen dokumentum nem tér ki, kizárólag azokat mutatja be, amelyeket a házi feladat megírásához saját kézzel szükségeltetett elékészíteni.

Modulok részletes ismertetése

RGB2Y modul

A modul feladata a HDMI vevőtől érkező RGB képadatok szürkeárnyalatossá alakítása. A megvalósítás az ITU-R BT.709 szabvány szerinti súlyozást használja, vagyis a vörös, zöld és kék komponensek nem azonos mértékben járulnak hozzá a kimeneti fényességértékhez. A hardveres implementáció során két DSP48E1 blokk került alkalmazásra, mellyel hatékonyan megvalósítható az Y értékre való konverzió. Az együtthatók fixpontos formában szerepelnek, a modul kimenetén pedig szaturáció biztosítja, hogy az eredmény 8 bites pixelértékként biztonságosan továbbadható legyen. Mivel a modul pipeline felépítésű, a képadat és a hozzá tartozó vezérlőjelek késleltetését össze kellett hangolni, ezért a valid, hsync és vsync jelek az adatúttal azonos mértékben késleltetve jelennek meg a kimeneten.

A modul órajelben mért késleltetése: 6 órajel.

Modul Verilog kódja

module rgb2y(

input                clk,

input  signed [17:0] kr_i,

input  signed [17:0] kb_i,

input                dv_i,

input                hs_i,

input                vs_i,

input          [7:0] r_i,

input          [7:0] g_i,

input          [7:0] b_i,

output               dv_o,

output               hs_o,

output               vs_o,

output         [7:0] y_o

);

integer i;

reg [7:0] g_dl[2:0];

// The green component is added back only after the first DSP stage,

// so it must be delayed to match the partial-product pipeline.

always @ (posedge clk)

for (i=0; i<3; i=i+1)

g_dl[i] <= (i==0) ? g_i : g_dl[i-1];

wire signed [47:0] p_o[1:0];

dsp dsp_0(

.clk_i  (clk),

.k_i    (kr_i),

.pa0_i  (r_i),

.pa1_i  (g_i),

// G must be aligned to the same Q17 fixed-point scale as the DSP products.

.pc_i   ({23'b0, g_dl[2], 17'b0}),

.p_o    (p_o[0])

);

dsp dsp_1(

.clk_i (clk),

.k_i   (kb_i),

.pa0_i (b_i),

.pa1_i (g_i),

.pc_i  (p_o[0]),

.p_o   (p_o[1])

);

reg [7:0] sat;

always @ (posedge clk)

if (p_o[1][47])

sat <= 8'b0;

else

// The fixed-point result is Q17-scaled, so bits [24:17] carry the

// 8-bit grayscale value once negative results have been clamped.

sat <= p_o[1][24:17];

assign y_o = sat;

reg [5:0] hs_dl;

reg [5:0] vs_dl;

reg [5:0] dv_dl;

// Keep the control signals aligned with the 6-cycle data path latency

// of the RGB-to-luma conversion, including the final saturation register.

always @ (posedge clk)

for (i=0; i<6; i=i+1)

begin

hs_dl[i] <= (i==0) ? hs_i : hs_dl[i-1];

vs_dl[i] <= (i==0) ? vs_i : vs_dl[i-1];

dv_dl[i] <= (i==0) ? dv_i : dv_dl[i-1];

end

assign hs_o = hs_dl[5];

assign vs_o = vs_dl[5];

assign dv_o = dv_dl[5];

endmodule

Szimulációs eredmények

Ennek a modulnak a működését legkönnyebben egy olyan szimulációval lehet ellenőrizni, amelyben a bemenetre egy színes kép pixeleit küldjük és azt az átalakítás után visszaírjuk egy file-ba és ezután összehasonlítjuk azokat. A kimenetnek az eredeti kép fekete-fehér megfelelőjének kell legyen.

BRAM sorbuffer-lánc

A bram_line_buffer modul feladata, hogy a beérkező szürkeárnyalatos pixeláramból előállítsa az aktuális pixelhez tartozó, függőleges irányban egymás alatt elhelyezkedő mintákat a 5×5 FIR szűrő számára. A megvalósítás négy egymás után kapcsolt, egy-egy képsort tároló BRAM-alapú ciklikus pufferre épül, amelyek read-first működési módban üzemelnek, így az adott címen korábban eltárolt pixel jelenik meg a kimeneten, mielőtt az új adat ugyanarra a címre beírásra kerülne. A pufferek azonos oszlopindexhez tartozó, megfelelően késleltetett címvezetéket kapnak, ezért a soronként eltolt adatok helyesen illeszkednek egymáshoz. Érvénytelen képpont esetén a modul nullát ír a memóriába, így a blank szakaszok nem visznek be hasznos képinformációt a pufferláncba. A memóriák induló tartalmát egy initial blokk nullázza, ezért a rendszer kezdeti állapotban is definiált értékekkel működik. A vezérlőjelek késleltetése a pufferlánc adatútjához igazodik, ezért a pixel_valid, h_sync és v_sync jelek a kimeneten megfelelően késleltetve jelennek meg.

A modul órajelben mért késleltetése (a vezérlő jelek alapján) 4 órajel.

Modul Verilog kódja

/*

* Single-line storage primitive.

*/

module bram_line_buffer_ram#(

parameter DATA_WIDTH = 8,

parameter LINE_WIDTH = 2048,

parameter ADDR_WIDTH = 11

)(

input  wire                  clk,

input  wire                  en,

input  wire [ADDR_WIDTH-1:0] addr,

input  wire [DATA_WIDTH-1:0] din,

output reg  [DATA_WIDTH-1:0] dout

);

reg [DATA_WIDTH-1:0] mem [0:LINE_WIDTH-1];

integer init_idx;

/*

* Initially set BRAMs to 0 for deterministic behaviour

*/

initial begin

for (init_idx = 0; init_idx < LINE_WIDTH; init_idx = init_idx + 1)

mem[init_idx] = {DATA_WIDTH{1'b0}};

dout = {DATA_WIDTH{1'b0}};

end

always @(posedge clk) begin

if (en) begin

// Read First mode

dout      <= mem[addr];

mem[addr] <= din;

end

end

endmodule

/*

* Four cascaded line memories store the previous 4 image rows.

*

* The visible outputs are delayed so that current_pixel and the four buffered

* lines all correspond to the same column when the FIR consumes them.

*/

module bram_line_buffer#(

parameter DATA_WIDTH = 8,

parameter LINE_WIDTH = 2048,

parameter ADDR_WIDTH = 11

)(

input  wire                  clk,

input  wire                  rst,

input  wire [DATA_WIDTH-1:0] pixel_in,

input  wire                  pixel_valid,

input  wire                  h_sync,

input  wire                  v_sync,

output wire [DATA_WIDTH-1:0] current_pixel,

output wire [DATA_WIDTH-1:0] line_1_pixel,

output wire [DATA_WIDTH-1:0] line_2_pixel,

output wire [DATA_WIDTH-1:0] line_3_pixel,

output wire [DATA_WIDTH-1:0] line_4_pixel,

output wire                  pixel_valid_out,

output wire                  h_sync_out,

output wire                  v_sync_out

);

reg  [ADDR_WIDTH-1:0] column_addr;

wire [ADDR_WIDTH-1:0] addr_current;

wire                  write_enable;

wire [DATA_WIDTH-1:0] pixel_or_black;

// The BRAM chain advances every clock; blanked pixels are written as zeros.

assign write_enable = 1'b1;

assign pixel_or_black = pixel_valid ? pixel_in : {DATA_WIDTH{1'b0}};

reg  [ADDR_WIDTH-1:0] addr_delayed1;

reg  [ADDR_WIDTH-1:0] addr_delayed2;

reg  [ADDR_WIDTH-1:0] addr_delayed3;

reg                   write_enable_delayed1;

reg                   write_enable_delayed2;

reg                   write_enable_delayed3;

reg                   pixel_valid_d1;

reg                   pixel_valid_d2;

reg                   pixel_valid_d3;

reg                   pixel_valid_d4;

reg                   h_sync_d1;

reg                   h_sync_d2;

reg                   h_sync_d3;

reg                   h_sync_d4;

reg                   h_sync_prev;

reg                   v_sync_d1;

reg                   v_sync_d2;

reg                   v_sync_d3;

reg                   v_sync_d4;

// Raw outputs of the four BRAM stages before final tap alignment.

wire [DATA_WIDTH-1:0] line_1_bram_pixel;

wire [DATA_WIDTH-1:0] line_2_bram_pixel;

wire [DATA_WIDTH-1:0] line_3_bram_pixel;

wire [DATA_WIDTH-1:0] line_4_bram_pixel;

// Internal delay chain for aligning the current input pixel to the deepest tap.

reg  [DATA_WIDTH-1:0] current_pixel_delayed1;

reg  [DATA_WIDTH-1:0] current_pixel_delayed2;

reg  [DATA_WIDTH-1:0] current_pixel_delayed3;

reg  [DATA_WIDTH-1:0] current_pixel_delayed4;

// Extra alignment registers for the shallower buffered lines.

reg  [DATA_WIDTH-1:0] line_1_pixel_delayed1;

reg  [DATA_WIDTH-1:0] line_1_pixel_delayed2;

reg  [DATA_WIDTH-1:0] line_1_pixel_delayed3;

reg  [DATA_WIDTH-1:0] line_2_pixel_delayed1;

reg  [DATA_WIDTH-1:0] line_2_pixel_delayed2;

reg  [DATA_WIDTH-1:0] line_3_pixel_delayed1;

wire                  start_of_line;

// Restart the circular addressing from column 0 on each new line.

assign start_of_line = h_sync & ~h_sync_prev;

assign addr_current = start_of_line ? {ADDR_WIDTH{1'b0}} : column_addr;

always @(posedge clk) begin

if (rst) begin

column_addr     <= {ADDR_WIDTH{1'b0}};

addr_delayed1   <= {ADDR_WIDTH{1'b0}};

addr_delayed2   <= {ADDR_WIDTH{1'b0}};

addr_delayed3   <= {ADDR_WIDTH{1'b0}};

write_enable_delayed1  <= 1'b0;

write_enable_delayed2  <= 1'b0;

write_enable_delayed3  <= 1'b0;

pixel_valid_d1  <= 1'b0;

pixel_valid_d2  <= 1'b0;

pixel_valid_d3  <= 1'b0;

pixel_valid_d4  <= 1'b0;

h_sync_d1       <= 1'b0;

h_sync_d2       <= 1'b0;

h_sync_d3       <= 1'b0;

h_sync_d4       <= 1'b0;

h_sync_prev     <= 1'b0;

v_sync_d1       <= 1'b0;

v_sync_d2       <= 1'b0;

v_sync_d3       <= 1'b0;

v_sync_d4       <= 1'b0;

current_pixel_delayed1 <= {DATA_WIDTH{1'b0}};

current_pixel_delayed2 <= {DATA_WIDTH{1'b0}};

current_pixel_delayed3 <= {DATA_WIDTH{1'b0}};

current_pixel_delayed4 <= {DATA_WIDTH{1'b0}};

line_1_pixel_delayed1 <= {DATA_WIDTH{1'b0}};

line_1_pixel_delayed2 <= {DATA_WIDTH{1'b0}};

line_1_pixel_delayed3 <= {DATA_WIDTH{1'b0}};

line_2_pixel_delayed1 <= {DATA_WIDTH{1'b0}};

line_2_pixel_delayed2 <= {DATA_WIDTH{1'b0}};

line_3_pixel_delayed1 <= {DATA_WIDTH{1'b0}};

end else begin

h_sync_prev <= h_sync;

if (start_of_line) begin

column_addr <= {{(ADDR_WIDTH-1){1'b0}}, 1'b1};

end else if (write_enable) begin

if (column_addr == LINE_WIDTH - 1) begin

column_addr <= {ADDR_WIDTH{1'b0}};

end else begin

column_addr <= column_addr + 1'b1;

end

end

// Each later BRAM stage reads the same column one cycle later.

addr_delayed1  <= addr_current;

addr_delayed2  <= addr_delayed1;

addr_delayed3  <= addr_delayed2;

// The delayed enables keep the BRAM pipeline aligned with read-first latency.

write_enable_delayed1 <= write_enable;

write_enable_delayed2 <= write_enable_delayed1;

write_enable_delayed3 <= write_enable_delayed2;

pixel_valid_d1 <= pixel_valid;

pixel_valid_d2 <= pixel_valid_d1;

pixel_valid_d3 <= pixel_valid_d2;

pixel_valid_d4 <= pixel_valid_d3;

h_sync_d1      <= h_sync;

h_sync_d2      <= h_sync_d1;

h_sync_d3      <= h_sync_d2;

h_sync_d4      <= h_sync_d3;

v_sync_d1      <= v_sync;

v_sync_d2      <= v_sync_d1;

v_sync_d3      <= v_sync_d2;

v_sync_d4      <= v_sync_d3;

// Match the current input pixel to the latency of the deepest line output.

current_pixel_delayed1 <= pixel_or_black;

current_pixel_delayed2 <= current_pixel_delayed1;

current_pixel_delayed3 <= current_pixel_delayed2;

current_pixel_delayed4 <= current_pixel_delayed3;

// The buffered rows need progressively fewer extra delay stages.

line_1_pixel_delayed1 <= line_1_bram_pixel;

line_1_pixel_delayed2 <= line_1_pixel_delayed1;

line_1_pixel_delayed3 <= line_1_pixel_delayed2;

line_2_pixel_delayed1 <= line_2_bram_pixel;

line_2_pixel_delayed2 <= line_2_pixel_delayed1;

line_3_pixel_delayed1 <= line_3_bram_pixel;

end

end

bram_line_buffer_ram #(

.DATA_WIDTH(DATA_WIDTH),

.LINE_WIDTH(LINE_WIDTH),

.ADDR_WIDTH(ADDR_WIDTH)

) bram_line_0 (

.clk(clk),

.en(write_enable),

.addr(addr_current),

.din(pixel_or_black),

.dout(line_1_bram_pixel)

);

bram_line_buffer_ram #(

.DATA_WIDTH(DATA_WIDTH),

.LINE_WIDTH(LINE_WIDTH),

.ADDR_WIDTH(ADDR_WIDTH)

) bram_line_1 (

.clk(clk),

.en(write_enable_delayed1),

.addr(addr_delayed1),

.din(line_1_bram_pixel),

.dout(line_2_bram_pixel)

);

bram_line_buffer_ram #(

.DATA_WIDTH(DATA_WIDTH),

.LINE_WIDTH(LINE_WIDTH),

.ADDR_WIDTH(ADDR_WIDTH)

) bram_line_2 (

.clk(clk),

.en(write_enable_delayed2),

.addr(addr_delayed2),

.din(line_2_bram_pixel),

.dout(line_3_bram_pixel)

);

bram_line_buffer_ram #(

.DATA_WIDTH(DATA_WIDTH),

.LINE_WIDTH(LINE_WIDTH),

.ADDR_WIDTH(ADDR_WIDTH)

) bram_line_3 (

.clk(clk),

.en(write_enable_delayed3),

.addr(addr_delayed3),

.din(line_3_bram_pixel),

.dout(line_4_bram_pixel)

);

// Output alignment:

//   current_pixel -> 4 extra cycles

//   line_1_pixel  -> 3 extra cycles

//   line_2_pixel  -> 2 extra cycles

//   line_3_pixel  -> 1 extra cycle

//   line_4_pixel  -> no extra delay

assign current_pixel    = current_pixel_delayed4;

assign line_1_pixel     = line_1_pixel_delayed3;

assign line_2_pixel     = line_2_pixel_delayed2;

assign line_3_pixel     = line_3_pixel_delayed1;

assign line_4_pixel     = line_4_bram_pixel;

// Control signals are delayed by the same 4 cycles as the visible data path.

assign pixel_valid_out  = pixel_valid_d4;

assign h_sync_out       = h_sync_d4;

assign v_sync_out       = v_sync_d4;

endmodule

Szimulációs eredmények

A szimulációhoz kihasználtam, hogy paraméteresen építettem fel a láncban szerepelő BRAM-okat, így kisebb, szimulációban szemmel jobban követhető eseteket is tudtam vizsgálni. A BRAM-ok szín kódolva vannak, miszerint az aktuális sor zöld, az azt megelőző rózsaszín, az azt megelőző narancs, az azt megelőző kék és az azt megelőző lila színnel van megjelölve. Ebben a szimulációban a v-sync nem jelenik meg, mert a buffer feltöltésének és adatmegjelenítésében ez nem játszik szerepet.

Ami látható, hogy 4 órajellel később jelenek meg az adatok a BRAM sorok kimenetén, ehhez megfelelően hozzá vannak igazítva a valid és h-sync jelek és a sorok is egymáshoz vannak igazítva.

2D 5x5 FIR szűrő

A FIR_filter_5x5 modul feladata, hogy a sorpufferből érkező 5×5 pixeles ablakon elvégezze a kétdimenziós FIR szűrést. A megvalósítás szisztolikus elrendezésű, vagyis a pixelek soronként, csővezetékesen haladnak végig az egyes szorzó-összegző egységeken, ami jól illeszkedik a folyamatos streaming feldolgozáshoz. Az aktuális mintapontnak a 5×5 ablak jobb alsó sarkát választottam, mert ez kauzális feldolgozást tesz lehetővé: a kimeneti érték előállításához így csak az aktuálisan rendelkezésre álló és korábban már beolvasott pixelekre van szükség. A szűrés során először mind az öt sorban külön-külön részösszeg képződik, majd ezek a sorösszegek a modul végén összeadásra kerülnek. Matematikailag a kimenet a következő alakban írható fel:

|  | Y = i=04j=04pijcij |
| --- | --- |

ahol pij az aktuális 5×5 ablak pixeleit, cij pedig a hozzájuk tartozó szűrőegyütthatókat jelöli. Mivel a rendszer a HDMI vevő által szolgáltatott rx_clk pixelórán működik, aktív képtartományban minden órajelciklusban egy új pixelt kell feldolgozni, ezért a teljesen streaming architektúrához a 25 szorzás/pixel igény miatt legalább 25 DSP blokk szükséges. A modul végén a felhalmozott eredmény jobbra shifteléssel kerül visszaskálázásra, így a tört részek elhagyhatók, majd szaturáció biztosítja, hogy a HDMI továbbítás felé csak 0...255 közé eső, 8 bites egész pixelérték kerüljön.

A modul órajelben mért késleltetése: 12 órajel.

BRAM 1 pixel

BRAM 2 pixel

BRAM 3 pixel

BRAM 4 pixel

current pixel

Modul Verilog kódja

/*

* DSP with passthrough for systolic mode

*/

module dsp_25x18_passthrough

#(

parameter integer A_REG = 2,

parameter integer B_REG = 2

)(

input  wire                     clk,

input  wire signed [24:0]       a,

input  wire signed [17:0]       b,

input  wire signed [47:0]       pci,

output wire signed [24:0]       a_passthrough,

output wire signed [47:0]       p

);

reg signed [24:0] a_reg [0:A_REG-1];

reg signed [17:0] b_reg [0:B_REG-1];

reg signed [42:0] m_reg;

reg signed [47:0] p_reg;

integer i;

integer init_idx;

/*

* For deterministic behaviour set registers to 0 initially

*/

initial begin

for (init_idx = 0; init_idx < A_REG; init_idx = init_idx + 1)

a_reg[init_idx] = 25'sd0;

for (init_idx = 0; init_idx < B_REG; init_idx = init_idx + 1)

b_reg[init_idx] = 18'sd0;

m_reg = 43'sd0;

p_reg = 48'sd0;

end

/*

* Parameterized registering, this is for easier object marking

* for the systolic FIR

* A is for the data

* B is for the coeff

*/

always @(posedge clk)

for (i = 0; i < A_REG; i = i + 1)

a_reg[i] <= (i == 0) ? a : a_reg[i-1];

always @(posedge clk)

for (i = 0; i < B_REG; i = i + 1)

b_reg[i] <= (i == 0) ? b : b_reg[i-1];

always @(posedge clk) begin

m_reg <= a_reg[A_REG-1] * b_reg[B_REG-1];

p_reg <= m_reg + pci;

end

assign a_passthrough = a_reg[A_REG-1];

assign p = p_reg;

endmodule

/*

* 1 row of systolic Fir filetr, 1D

*/

module fir_row_5_cells

(

input  wire                     clk,

input  wire signed [24:0]       pixel_in,

input  wire signed [17:0]       coeff_0_i,

input  wire signed [17:0]       coeff_1_i,

input  wire signed [17:0]       coeff_2_i,

input  wire signed [17:0]       coeff_3_i,

input  wire signed [17:0]       coeff_4_i,

input  wire signed [47:0]       pci_i,

output wire signed [47:0]       p_o

);

localparam integer COEFF_REG_STAGES = 1;

wire signed [24:0] pixel_0_to_1;

wire signed [24:0] pixel_1_to_2;

wire signed [24:0] pixel_2_to_3;

wire signed [24:0] pixel_3_to_4;

wire signed [24:0] pixel_4_unused;

wire signed [47:0] p_0;

wire signed [47:0] p_1;

wire signed [47:0] p_2;

wire signed [47:0] p_3;

wire signed [47:0] p_4;

dsp_25x18_passthrough #(

.A_REG(1),

.B_REG(COEFF_REG_STAGES)

) dsp_cell_0 (

.clk(clk),

.a(pixel_in),

.b(coeff_0_i),

.pci(pci_i),

.a_passthrough(pixel_0_to_1),

.p(p_0)

);

dsp_25x18_passthrough #(

.A_REG(2),

.B_REG(COEFF_REG_STAGES)

) dsp_cell_1 (

.clk(clk),

.a(pixel_0_to_1),

.b(coeff_1_i),

.pci(p_0),

.a_passthrough(pixel_1_to_2),

.p(p_1)

);

dsp_25x18_passthrough #(

.A_REG(2),

.B_REG(COEFF_REG_STAGES)

) dsp_cell_2 (

.clk(clk),

.a(pixel_1_to_2),

.b(coeff_2_i),

.pci(p_1),

.a_passthrough(pixel_2_to_3),

.p(p_2)

);

dsp_25x18_passthrough #(

.A_REG(2),

.B_REG(COEFF_REG_STAGES)

) dsp_cell_3 (

.clk(clk),

.a(pixel_2_to_3),

.b(coeff_3_i),

.pci(p_2),

.a_passthrough(pixel_3_to_4),

.p(p_3)

);

dsp_25x18_passthrough #(

.A_REG(2),

.B_REG(COEFF_REG_STAGES)

) dsp_cell_4 (

.clk(clk),

.a(pixel_3_to_4),

.b(coeff_4_i),

.pci(p_3),

.a_passthrough(pixel_4_unused),

.p(p_4)

);

assign p_o = p_4;

endmodule

/*

* full 5x5 window from the 5 rows

*/

module FIR_filter_5x5 #(

parameter integer CTRL_DELAY = 12

)(

input  wire                     clk,

input  wire                     pixel_valid_i,

input  wire                     h_sync_i,

input  wire                     v_sync_i,

input  wire signed [24:0]       pixel_0_i,

input  wire signed [24:0]       pixel_1_i,

input  wire signed [24:0]       pixel_2_i,

input  wire signed [24:0]       pixel_3_i,

input  wire signed [24:0]       pixel_4_i,

input  wire signed [15:0]       coeff_00_i,

input  wire signed [15:0]       coeff_01_i,

input  wire signed [15:0]       coeff_02_i,

input  wire signed [15:0]       coeff_03_i,

input  wire signed [15:0]       coeff_04_i,

input  wire signed [15:0]       coeff_10_i,

input  wire signed [15:0]       coeff_11_i,

input  wire signed [15:0]       coeff_12_i,

input  wire signed [15:0]       coeff_13_i,

input  wire signed [15:0]       coeff_14_i,

input  wire signed [15:0]       coeff_20_i,

input  wire signed [15:0]       coeff_21_i,

input  wire signed [15:0]       coeff_22_i,

input  wire signed [15:0]       coeff_23_i,

input  wire signed [15:0]       coeff_24_i,

input  wire signed [15:0]       coeff_30_i,

input  wire signed [15:0]       coeff_31_i,

input  wire signed [15:0]       coeff_32_i,

input  wire signed [15:0]       coeff_33_i,

input  wire signed [15:0]       coeff_34_i,

input  wire signed [15:0]       coeff_40_i,

input  wire signed [15:0]       coeff_41_i,

input  wire signed [15:0]       coeff_42_i,

input  wire signed [15:0]       coeff_43_i,

input  wire signed [15:0]       coeff_44_i,

output reg  [7:0]               fir_out_sat,

output wire                     pixel_valid_out,

output wire                     h_sync_out,

output wire                     v_sync_out

);

wire signed [47:0] row0_p;

wire signed [47:0] row1_p;

wire signed [47:0] row2_p;

wire signed [47:0] row3_p;

wire signed [47:0] row4_p;

wire signed [47:0] row_sum_total;

wire signed [17:0] coeff_00_ext;

wire signed [17:0] coeff_01_ext;

wire signed [17:0] coeff_02_ext;

wire signed [17:0] coeff_03_ext;

wire signed [17:0] coeff_04_ext;

wire signed [17:0] coeff_10_ext;

wire signed [17:0] coeff_11_ext;

wire signed [17:0] coeff_12_ext;

wire signed [17:0] coeff_13_ext;

wire signed [17:0] coeff_14_ext;

wire signed [17:0] coeff_20_ext;

wire signed [17:0] coeff_21_ext;

wire signed [17:0] coeff_22_ext;

wire signed [17:0] coeff_23_ext;

wire signed [17:0] coeff_24_ext;

wire signed [17:0] coeff_30_ext;

wire signed [17:0] coeff_31_ext;

wire signed [17:0] coeff_32_ext;

wire signed [17:0] coeff_33_ext;

wire signed [17:0] coeff_34_ext;

wire signed [17:0] coeff_40_ext;

wire signed [17:0] coeff_41_ext;

wire signed [17:0] coeff_42_ext;

wire signed [17:0] coeff_43_ext;

wire signed [17:0] coeff_44_ext;

/*

* Coeffs are 16 bit bit dsp B reg is 18, extend for deterministic behaviour

*/

assign coeff_00_ext = {{2{coeff_00_i[15]}}, coeff_00_i};

assign coeff_01_ext = {{2{coeff_01_i[15]}}, coeff_01_i};

assign coeff_02_ext = {{2{coeff_02_i[15]}}, coeff_02_i};

assign coeff_03_ext = {{2{coeff_03_i[15]}}, coeff_03_i};

assign coeff_04_ext = {{2{coeff_04_i[15]}}, coeff_04_i};

assign coeff_10_ext = {{2{coeff_10_i[15]}}, coeff_10_i};

assign coeff_11_ext = {{2{coeff_11_i[15]}}, coeff_11_i};

assign coeff_12_ext = {{2{coeff_12_i[15]}}, coeff_12_i};

assign coeff_13_ext = {{2{coeff_13_i[15]}}, coeff_13_i};

assign coeff_14_ext = {{2{coeff_14_i[15]}}, coeff_14_i};

assign coeff_20_ext = {{2{coeff_20_i[15]}}, coeff_20_i};

assign coeff_21_ext = {{2{coeff_21_i[15]}}, coeff_21_i};

assign coeff_22_ext = {{2{coeff_22_i[15]}}, coeff_22_i};

assign coeff_23_ext = {{2{coeff_23_i[15]}}, coeff_23_i};

assign coeff_24_ext = {{2{coeff_24_i[15]}}, coeff_24_i};

assign coeff_30_ext = {{2{coeff_30_i[15]}}, coeff_30_i};

assign coeff_31_ext = {{2{coeff_31_i[15]}}, coeff_31_i};

assign coeff_32_ext = {{2{coeff_32_i[15]}}, coeff_32_i};

assign coeff_33_ext = {{2{coeff_33_i[15]}}, coeff_33_i};

assign coeff_34_ext = {{2{coeff_34_i[15]}}, coeff_34_i};

assign coeff_40_ext = {{2{coeff_40_i[15]}}, coeff_40_i};

assign coeff_41_ext = {{2{coeff_41_i[15]}}, coeff_41_i};

assign coeff_42_ext = {{2{coeff_42_i[15]}}, coeff_42_i};

assign coeff_43_ext = {{2{coeff_43_i[15]}}, coeff_43_i};

assign coeff_44_ext = {{2{coeff_44_i[15]}}, coeff_44_i};

/*

* Delay line for valid and sync

*/

generate

if (CTRL_DELAY > 0) begin : gen_ctrl_delay

reg valid_delay [0:CTRL_DELAY-1];

reg hsync_delay [0:CTRL_DELAY-1];

reg vsync_delay [0:CTRL_DELAY-1];

integer ctrl_idx;

always @(posedge clk) begin

for (ctrl_idx = 0; ctrl_idx < CTRL_DELAY; ctrl_idx = ctrl_idx + 1) begin

valid_delay[ctrl_idx] <= (ctrl_idx == 0) ? pixel_valid_i : valid_delay[ctrl_idx-1];

hsync_delay[ctrl_idx] <= (ctrl_idx == 0) ? h_sync_i      : hsync_delay[ctrl_idx-1];

vsync_delay[ctrl_idx] <= (ctrl_idx == 0) ? v_sync_i      : vsync_delay[ctrl_idx-1];

end

end

assign pixel_valid_out = valid_delay[CTRL_DELAY-1];

assign h_sync_out      = hsync_delay[CTRL_DELAY-1];

assign v_sync_out      = vsync_delay[CTRL_DELAY-1];

end else begin : gen_ctrl_passthrough

assign pixel_valid_out = pixel_valid_i;

assign h_sync_out      = h_sync_i;

assign v_sync_out      = v_sync_i;

end

endgenerate

fir_row_5_cells row_0 (

.clk(clk),

.pixel_in(pixel_0_i),

.coeff_0_i(coeff_00_ext),

.coeff_1_i(coeff_01_ext),

.coeff_2_i(coeff_02_ext),

.coeff_3_i(coeff_03_ext),

.coeff_4_i(coeff_04_ext),

.pci_i(48'sd0),

.p_o(row0_p)

);

fir_row_5_cells row_1 (

.clk(clk),

.pixel_in(pixel_1_i),

.coeff_0_i(coeff_10_ext),

.coeff_1_i(coeff_11_ext),

.coeff_2_i(coeff_12_ext),

.coeff_3_i(coeff_13_ext),

.coeff_4_i(coeff_14_ext),

.pci_i(48'sd0),

.p_o(row1_p)

);

fir_row_5_cells row_2 (

.clk(clk),

.pixel_in(pixel_2_i),

.coeff_0_i(coeff_20_ext),

.coeff_1_i(coeff_21_ext),

.coeff_2_i(coeff_22_ext),

.coeff_3_i(coeff_23_ext),

.coeff_4_i(coeff_24_ext),

.pci_i(48'sd0),

.p_o(row2_p)

);

fir_row_5_cells row_3 (

.clk(clk),

.pixel_in(pixel_3_i),

.coeff_0_i(coeff_30_ext),

.coeff_1_i(coeff_31_ext),

.coeff_2_i(coeff_32_ext),

.coeff_3_i(coeff_33_ext),

.coeff_4_i(coeff_34_ext),

.pci_i(48'sd0),

.p_o(row3_p)

);

fir_row_5_cells row_4 (

.clk(clk),

.pixel_in(pixel_4_i),

.coeff_0_i(coeff_40_ext),

.coeff_1_i(coeff_41_ext),

.coeff_2_i(coeff_42_ext),

.coeff_3_i(coeff_43_ext),

.coeff_4_i(coeff_44_ext),

.pci_i(48'sd0),

.p_o(row4_p)

);

// First the five horizontal FIR rows are evaluated independently,

// then their partial sums are accumulated into the final 5x5 result.

assign row_sum_total = row0_p + row1_p + row2_p + row3_p + row4_p;

// Drop the fractional Q8.8 part before clamping the result to 8-bit range.

always @(posedge clk) begin

if (row_sum_total[47]) begin

fir_out_sat <= 8'd0;

end else if (|row_sum_total[46:16]) begin

fir_out_sat <= 8'd255;

end else begin

fir_out_sat <= row_sum_total[15:8];

end

end

endmodule

Szimulációs eredmények

Mivel a BRAM-ot előzőleg teszteltük és az RGB2Y modul is rendeltetés szerűén üzemelt, ezért a FIR szűrőt tesztelhettem már képi bemenettel is. Itt ugyan azt a Lena képet használtam, mint az RGB2Y modul esetén és 3 jólismert, szemmel is könnyen ellenőrizhető szűrők együtthatókészletét választottam. Ezek a Gauss, a Laplace szűrők és egy olyan együtthatókészlet, ami csak átenged. Ennek megfelelően az áteresztő szűrő szerepel az első képen, mellette jobbra a Gauss-szűrt (ennek tulajdonsága, hogy elmossa a képet), illetve utolsóként a Laplace szűrő (ez pedig egy éldetektorként funkcionál).

Amit különösen meg kell jegyezni az a FIR szűrő és a blank periódusok közötti összefüggés. Mivel ilyenkor 0 értékekkel kerülnek feltöltésre a BRAM pufferek, ezért az első 4 sor és első 4 oszlop minden esetben fekete pixelekkel lesz feltöltve. Ez azonban egy 1600*900-as felbontás mellett elhanyagolható.

A FIR szűrőt önmagában a BRAM sorbufferekkel is teszteltem. Ebben a tesztben az 5x5 ablak együtthatóit úgy állítottam be, hogy a jobb legszélső oszlop elemei 1, a többi 0 legyen. Ezzel (és tudva azt, hogy az ablak a jobb alsó sarkos igazítással számol) ezt értjük el, hogy a FIR szűrő kimenetén a sorokban oszlopról oszlopra haladva az elemek összegét látjuk, például a jelölt 18 = 17+1.

Processzoros alrendszer

A képfeldolgozó adatút mellett a rendszer egy külön processzoros alrendszert is tartalmaz, amelynek feladata a FIR szűrő együtthatóinak futásidejű kezelése. Ez az alrendszer egy MicroBlaze RISC-V processzorra épül, amely helyi memóriával, UART perifériával és egy egyedi AXI Lite csatolású együttható-regiszterbankkal kapcsolódik a rendszerhez. A processzor a soros porton keresztül fogadja a felhasználó által megadott szűrőegyütthatókat, majd ezeket az AXI buszon keresztül beírja a hardveres FIR szűrőhöz kapcsolódó regiszterekbe. Ennek köszönhetően a szűrőmaszk futás közben, újraszintetizálás nélkül is módosítható.

Blokkvázlat

A processzoros alrendszer központi eleme a MicroBlaze RISC-V processzor, amely egy helyi BRAM alapú memóriát használ a program és az adatok tárolására. A processzor AXI buszon keresztül két perifériát ér el: az egyik az AXI UARTLite, amely a soros kommunikációt biztosítja, a másik pedig a saját fejlesztésű fir_coeff_bank_axi periféria, amely 25 darab, egyenként 16 bites FIR együttható tárolását teszi lehetővé. A rendszer órajelét a processzoros rész számára egy 100 MHz-es órajel biztosítja. A blokkdiagramon jól látható, hogy a processzoros alrendszer nem közvetlenül a képfeldolgozó adatút része, hanem annak paraméterezését végzi. Ezen rendszerből szintetizált wrapper modul kerül példányosításra a top modulban, ahol a 400 bit széles (25*16 bit) coeff_out adatvonal szétdarabolóik és a a megfelelő 16 széles csoportosítások a megfelelő bemeneteire kerülnek a FIR-szűrőnek.

Címkiosztás

A processzor címtartományában a helyi memória és a két AXI periféria külön címteret kapott. A helyi BRAM a 0x00000000 - 0x0000FFFF tartományban található, amelyet a processzor utasítás- és adatmemóriaként használ. Az AXI UARTLite periféria a 0x40600000 - 0x4060FFFF címtartományra van leképezve, ezen keresztül történik a soros kommunikáció a felhasználóval. A saját fejlesztésű fir_coeff_bank_axi periféria a 0x44A00000 - 0x44A0FFFF tartományban helyezkedik el. A szoftver a szűrőegyütthatókat ebbe a regiszertérbe írja be, így a hardveres FIR szűrő futás közben új paraméterekkel működtethető.

Implementációs eredmények

Az implementációs eredmények alapján a terv a célhardveren sikeresen megvalósítható, és az időzítési követelményeket is teljesíti: a tervezői környezet nem jelzett sem failed route, sem timing hibát. A routed timing summary szerint a legrosszabb setup slack 1.986 ns, míg a legrosszabb hold slack 0.052 ns, tehát a rendszer időzítési szempontból kivitelezhető.

Az erőforrás-felhasználás alapján az implementált rendszer mérsékelt méretűnek tekinthető. Az implementáció után a terv összesen 3564 LUT-ot, 3875 flip-flopot, 19 BRAM blokkot és 29 DSP egységet használ. A DSP-felhasználásból az egészet az 5×5 FIR szűrő és a szürkeárnyalatos előfeldolgozás igényli, míg a BRAM blokkokat elsősorban a sorpufferek és a processzoros alrendszer helyi memóriája használja.

Teljes rendszer (top_module)

module hdmi_top(

input  wire       clk100M,

input  wire       rstbt,

output wire [7:0] led_r,

input  wire [7:0] sw,

inout  wire [3:0] bt,

input  wire       uart_rxd,

output wire       uart_txd,

input  wire       hdmi_rx_d0_p,

input  wire       hdmi_rx_d0_n,

input  wire       hdmi_rx_d1_p,

input  wire       hdmi_rx_d1_n,

input  wire       hdmi_rx_d2_p,

input  wire       hdmi_rx_d2_n,

input  wire       hdmi_rx_clk_p,

input  wire       hdmi_rx_clk_n,

input  wire       hdmi_rx_cec,

output wire       hdmi_rx_hpd,

input  wire       hdmi_rx_scl,

inout  wire       hdmi_rx_sda,

output wire       hdmi_tx_d0_p,

output wire       hdmi_tx_d0_n,

output wire       hdmi_tx_d1_p,

output wire       hdmi_tx_d1_n,

output wire       hdmi_tx_d2_p,

output wire       hdmi_tx_d2_n,

output wire       hdmi_tx_clk_p,

output wire       hdmi_tx_clk_n,

input  wire       hdmi_tx_cec,

input  wire       hdmi_tx_hpdn,

input  wire       hdmi_tx_scl,

input  wire       hdmi_tx_sda

);

//******************************************************************************

//* Generating the 200 MHz reference clock for the IDELAYCTRL.                 *

//******************************************************************************

wire clk200M;

wire pll_clkfb;

wire pll_locked;

PLLE2_BASE #(

.BANDWIDTH("OPTIMIZED"),         // OPTIMIZED, HIGH, LOW

.CLKFBOUT_MULT(10),              // Multiply value for all CLKOUT, (2-64)

.CLKFBOUT_PHASE(0.0),            // Phase offset in degrees of CLKFB, (-360.000-360.000).

.CLKIN1_PERIOD(1000.0 / 100.0),  // Input clock period in ns to ps resolution (i.e. 33.333 is 30 MHz).

.CLKOUT0_DIVIDE(5),              // CLKOUT0_DIVIDE - CLKOUT5_DIVIDE: Divide amount for each CLKOUT (1-128)

.CLKOUT1_DIVIDE(1),

.CLKOUT2_DIVIDE(1),

.CLKOUT3_DIVIDE(1),

.CLKOUT4_DIVIDE(1),

.CLKOUT5_DIVIDE(1),

.CLKOUT0_DUTY_CYCLE(0.5),        // CLKOUT0_DUTY_CYCLE - CLKOUT5_DUTY_CYCLE: Duty cycle for each CLKOUT (0.001-0.999).

.CLKOUT1_DUTY_CYCLE(0.5),

.CLKOUT2_DUTY_CYCLE(0.5),

.CLKOUT3_DUTY_CYCLE(0.5),

.CLKOUT4_DUTY_CYCLE(0.5),

.CLKOUT5_DUTY_CYCLE(0.5),

.CLKOUT0_PHASE(0.0),             // CLKOUT0_PHASE - CLKOUT5_PHASE: Phase offset for each CLKOUT (-360.000-360.000).

.CLKOUT1_PHASE(0.0),

.CLKOUT2_PHASE(0.0),

.CLKOUT3_PHASE(0.0),

.CLKOUT4_PHASE(0.0),

.CLKOUT5_PHASE(0.0),

.DIVCLK_DIVIDE(1),               // Master division value, (1-56)

.REF_JITTER1(0.0),               // Reference input jitter in UI, (0.000-0.999).

.STARTUP_WAIT("FALSE")           // Delay DONE until PLL Locks, ("TRUE"/"FALSE")

) clk_generator1 (

.CLKOUT0(clk200M),               // 1-bit output: CLKOUT0

.CLKOUT1(),                      // 1-bit output: CLKOUT1

.CLKOUT2(),                      // 1-bit output: CLKOUT2

.CLKOUT3(),                      // 1-bit output: CLKOUT3

.CLKOUT4(),                      // 1-bit output: CLKOUT4

.CLKOUT5(),                      // 1-bit output: CLKOUT5

.CLKFBOUT(pll_clkfb),            // 1-bit output: Feedback clock

.LOCKED(pll_locked),             // 1-bit output: LOCK

.CLKIN1(clk100M),                // 1-bit input: Input clock

.PWRDWN(1'b0),                   // 1-bit input: Power-down

.RST(rstbt),                     // 1-bit input: Reset

.CLKFBIN(pll_clkfb)              // 1-bit input: Feedback clock

);

wire rst;

assign rst = ~pll_locked;

wire clk_200M;

BUFG BUFG_200M (

.O(clk_200M),

.I(clk200M)

);

wire rx_clk, rx_clk_5x;

wire [7:0] rx_red, rx_green, rx_blue;

wire rx_dv, rx_hs, rx_vs;

wire [5:0] rx_status;

hdmi_rx hdmi_rx_0(

.clk_200M(clk_200M),

.rst(rst),

.hdmi_rx_cec(hdmi_rx_cec),

.hdmi_rx_hpd(hdmi_rx_hpd),

.hdmi_rx_scl(hdmi_rx_scl),

.hdmi_rx_sda(hdmi_rx_sda),

.hdmi_rx_clk_p(hdmi_rx_clk_p),

.hdmi_rx_clk_n(hdmi_rx_clk_n),

.hdmi_rx_d0_p(hdmi_rx_d0_p),

.hdmi_rx_d0_n(hdmi_rx_d0_n),

.hdmi_rx_d1_p(hdmi_rx_d1_p),

.hdmi_rx_d1_n(hdmi_rx_d1_n),

.hdmi_rx_d2_p(hdmi_rx_d2_p),

.hdmi_rx_d2_n(hdmi_rx_d2_n),

.rx_clk(rx_clk),

.rx_clk_5x(rx_clk_5x),

.rx_red(rx_red),

.rx_green(rx_green),

.rx_blue(rx_blue),

.rx_dv(rx_dv),

.rx_hs(rx_hs),

.rx_vs(rx_vs),

.rx_status(rx_status)

);

localparam integer FIR_LINE_WIDTH = 2048;

localparam integer FIR_ADDR_WIDTH = 11;

wire rgb2y_dv;

wire rgb2y_hs;

wire rgb2y_vs;

wire [7:0] rgb2y_y;

wire [399:0] coeff_wire;

wire signed [15:0] fir_coeff_00;

wire signed [15:0] fir_coeff_01;

wire signed [15:0] fir_coeff_02;

wire signed [15:0] fir_coeff_03;

wire signed [15:0] fir_coeff_04;

wire signed [15:0] fir_coeff_10;

wire signed [15:0] fir_coeff_11;

wire signed [15:0] fir_coeff_12;

wire signed [15:0] fir_coeff_13;

wire signed [15:0] fir_coeff_14;

wire signed [15:0] fir_coeff_20;

wire signed [15:0] fir_coeff_21;

wire signed [15:0] fir_coeff_22;

wire signed [15:0] fir_coeff_23;

wire signed [15:0] fir_coeff_24;

wire signed [15:0] fir_coeff_30;

wire signed [15:0] fir_coeff_31;

wire signed [15:0] fir_coeff_32;

wire signed [15:0] fir_coeff_33;

wire signed [15:0] fir_coeff_34;

wire signed [15:0] fir_coeff_40;

wire signed [15:0] fir_coeff_41;

wire signed [15:0] fir_coeff_42;

wire signed [15:0] fir_coeff_43;

wire signed [15:0] fir_coeff_44;

wire [7:0] current_pixel;

wire [7:0] line_1_pixel;

wire [7:0] line_2_pixel;

wire [7:0] line_3_pixel;

wire [7:0] line_4_pixel;

wire linebuf_dv;

wire linebuf_hs;

wire linebuf_vs;

wire signed [24:0] fir_pixel_0;

wire signed [24:0] fir_pixel_1;

wire signed [24:0] fir_pixel_2;

wire signed [24:0] fir_pixel_3;

wire signed [24:0] fir_pixel_4;

wire [7:0] fir_out_sat;

wire fir_dv;

wire fir_hs;

wire fir_vs;

wire [7:0] tx_red;

wire [7:0] tx_green;

wire [7:0] tx_blue;

wire tx_dv;

wire tx_hs;

wire tx_vs;

cpu_hdmi_system_sv processor_inst (

.rstbtn(rstbt),

.clk_in(clk100M),

.uart_rx(uart_rxd),

.uart_tx(uart_txd),

.coeff_out_0(coeff_wire)

);

assign fir_coeff_00 = coeff_wire[15:0];

assign fir_coeff_01 = coeff_wire[31:16];

assign fir_coeff_02 = coeff_wire[47:32];

assign fir_coeff_03 = coeff_wire[63:48];

assign fir_coeff_04 = coeff_wire[79:64];

assign fir_coeff_10 = coeff_wire[95:80];

assign fir_coeff_11 = coeff_wire[111:96];

assign fir_coeff_12 = coeff_wire[127:112];

assign fir_coeff_13 = coeff_wire[143:128];

assign fir_coeff_14 = coeff_wire[159:144];

assign fir_coeff_20 = coeff_wire[175:160];

assign fir_coeff_21 = coeff_wire[191:176];

assign fir_coeff_22 = coeff_wire[207:192];

assign fir_coeff_23 = coeff_wire[223:208];

assign fir_coeff_24 = coeff_wire[239:224];

assign fir_coeff_30 = coeff_wire[255:240];

assign fir_coeff_31 = coeff_wire[271:256];

assign fir_coeff_32 = coeff_wire[287:272];

assign fir_coeff_33 = coeff_wire[303:288];

assign fir_coeff_34 = coeff_wire[319:304];

assign fir_coeff_40 = coeff_wire[335:320];

assign fir_coeff_41 = coeff_wire[351:336];

assign fir_coeff_42 = coeff_wire[367:352];

assign fir_coeff_43 = coeff_wire[383:368];

assign fir_coeff_44 = coeff_wire[399:384];

assign fir_pixel_0 = {17'd0, current_pixel};

assign fir_pixel_1 = {17'd0, line_1_pixel};

assign fir_pixel_2 = {17'd0, line_2_pixel};

assign fir_pixel_3 = {17'd0, line_3_pixel};

assign fir_pixel_4 = {17'd0, line_4_pixel};

rgb2y rgb2y_0(

.clk(rx_clk),

.kr_i(18'sd27865),

.kb_i(18'sd9463),

.dv_i(rx_dv),

.hs_i(rx_hs),

.vs_i(rx_vs),

.r_i(rx_red),

.g_i(rx_green),

.b_i(rx_blue),

.dv_o(rgb2y_dv),

.hs_o(rgb2y_hs),

.vs_o(rgb2y_vs),

.y_o(rgb2y_y)

);

bram_line_buffer #(

.DATA_WIDTH(8),

.LINE_WIDTH(FIR_LINE_WIDTH),

.ADDR_WIDTH(FIR_ADDR_WIDTH)

) line_buffer_0 (

.clk(rx_clk),

.rst(rst),

.pixel_in(rgb2y_y),

.pixel_valid(rgb2y_dv),

.h_sync(rgb2y_hs),

.v_sync(rgb2y_vs),

.current_pixel(current_pixel),

.line_1_pixel(line_1_pixel),

.line_2_pixel(line_2_pixel),

.line_3_pixel(line_3_pixel),

.line_4_pixel(line_4_pixel),

.pixel_valid_out(linebuf_dv),

.h_sync_out(linebuf_hs),

.v_sync_out(linebuf_vs)

);

FIR_filter_5x5 #(

.CTRL_DELAY(8)

) fir_0 (

.clk(rx_clk),

.pixel_valid_i(linebuf_dv),

.h_sync_i(linebuf_hs),

.v_sync_i(linebuf_vs),

.pixel_0_i(fir_pixel_0),

.pixel_1_i(fir_pixel_1),

.pixel_2_i(fir_pixel_2),

.pixel_3_i(fir_pixel_3),

.pixel_4_i(fir_pixel_4),

.coeff_00_i(fir_coeff_00),

.coeff_01_i(fir_coeff_01),

.coeff_02_i(fir_coeff_02),

.coeff_03_i(fir_coeff_03),

.coeff_04_i(fir_coeff_04),

.coeff_10_i(fir_coeff_10),

.coeff_11_i(fir_coeff_11),

.coeff_12_i(fir_coeff_12),

.coeff_13_i(fir_coeff_13),

.coeff_14_i(fir_coeff_14),

.coeff_20_i(fir_coeff_20),

.coeff_21_i(fir_coeff_21),

.coeff_22_i(fir_coeff_22),

.coeff_23_i(fir_coeff_23),

.coeff_24_i(fir_coeff_24),

.coeff_30_i(fir_coeff_30),

.coeff_31_i(fir_coeff_31),

.coeff_32_i(fir_coeff_32),

.coeff_33_i(fir_coeff_33),

.coeff_34_i(fir_coeff_34),

.coeff_40_i(fir_coeff_40),

.coeff_41_i(fir_coeff_41),

.coeff_42_i(fir_coeff_42),

.coeff_43_i(fir_coeff_43),

.coeff_44_i(fir_coeff_44),

.fir_out_sat(fir_out_sat),

.pixel_valid_out(fir_dv),

.h_sync_out(fir_hs),

.v_sync_out(fir_vs)

);

assign tx_dv    = fir_dv;

assign tx_hs    = fir_hs;

assign tx_vs    = fir_vs;

assign tx_red   = fir_out_sat;

assign tx_green = fir_out_sat;

assign tx_blue  = fir_out_sat;

hdmi_tx hdmi_tx_0(

.tx_clk(rx_clk),

.tx_clk_5x(rx_clk_5x),

.rst(rst),

.tx_red(tx_red),

.tx_green(tx_green),

.tx_blue(tx_blue),

.tx_dv(tx_dv),

.tx_hs(tx_hs),

.tx_vs(tx_vs),

.hdmi_tx_clk_p(hdmi_tx_clk_p),

.hdmi_tx_clk_n(hdmi_tx_clk_n),

.hdmi_tx_d0_p(hdmi_tx_d0_p),

.hdmi_tx_d0_n(hdmi_tx_d0_n),

.hdmi_tx_d1_p(hdmi_tx_d1_p),

.hdmi_tx_d1_n(hdmi_tx_d1_n),

.hdmi_tx_d2_p(hdmi_tx_d2_p),

.hdmi_tx_d2_n(hdmi_tx_d2_n)

);

assign led_r = {pll_locked, 1'b0, rx_status};

endmodule

Szoftver leírás

A MicroBlaze szoftvere

A MicroBlaze szoftvere egy folyamatosan futó, soros kommunikációra épülő vezérlőprogram. Induláskor inicializálja az UART perifériát, majd betölti az alapértelmezett FIR együtthatókat a hardveres regiszterbankba. Ez az alapállapot egy áteresztő szűrőnek felel meg, ahol minden együttható nulla, kivéve a jobb alsó sarokhoz tartozó elemet, amely +1.0000 értéket kap. Ezt követően a program a soros vonalon érkező karaktereket figyeli, soronként értelmezi a felhasználói bemenetet, majd 25 darab együttható összegyűjtése után ezeket fixpontos formára alakítja és beírja az együttható bank AXI regisztereibe.

Folyamatábra

Megvalósított függvények

decimal4_to_q8_8_raw(): A felhasználó által megadott, négy tizedesjegyes decimális értéket Q8.8 fixpontos formára alakítja. Erre azért van szükség, mert a hardveres FIR szűrő együtthatói egész számként kerülnek beírásra a regiszterekbe, miközben törtrészes értékeket is kezelni kell.

load_default_coefficients(): Inicializálja a FIR együtthatókat. A függvény minden regisztert nullára állít, majd a jobb alsó sarokhoz tartozó együtthatót 1.0 értékre állítja, így a rendszer induláskor áteresztő szűrőként működik.

main(): A program fő vezérlőfüggvénye. Elvégzi az UART inicializálását, betölti az alapértelmezett együtthatókat, majd végtelen ciklusban fogadja és feldolgozza a felhasználó által beírt karaktereket. A függvény ellenőrzi a bemeneti formátumot, eltárolja a 25 darab együtthatót, majd azokat fixpontos formára alakítva beírja az AXI regiszterekbe.

A kódban egyébként megtalálható függvények, mint például az UART vezérlése vagy az együttható bankba való írás mind a Xilinx által generált Board Support Package részét képzik.

Kód

#include "xparameters.h"

#include "xil_io.h"

#include "xil_printf.h"

#include "xuartlite_l.h"

static int decimal4_to_q8_8_raw(int decimal4_value)

{

long long scaled;

/* Round the 4-decimal fixed-point input to the nearest Q8.8 integer. */

if (decimal4_value >= 0) {

scaled = (long long)decimal4_value * 256 + 5000;

} else {

scaled = (long long)decimal4_value * 256 - 5000;

}

return (int)(scaled / 10000);

}

static void load_default_coefficients(void)

{

int coeff_index;

/* Clear all 25 coefficient registers before loading the identity tap. */

for (coeff_index = 0; coeff_index < 25; ++coeff_index) {

Xil_Out32(XPAR_FIR_COEFF_BANK_AXI_0_BASEADDR + 4 * coeff_index, 0);

}

/*

* The streaming FIR uses the bottom-right tap as the "current" sample

* location, so an identity kernel is EH 24 = +1.0000.

*/

Xil_Out32(XPAR_FIR_COEFF_BANK_AXI_0_BASEADDR + 4 * 24, 256);

}

int main(void)

{

u8 received_byte;

char input_line[64];

int input_length = 0;

int decimal4_coefficients[25];

int stored_coefficients = 0;

int coeff_index;

XUartLite_SetControlReg(XPAR_XUARTLITE_0_BASEADDR,

XUL_CR_FIFO_RX_RESET | XUL_CR_FIFO_TX_RESET);

load_default_coefficients();

xil_printf("\r\nUART FIR coefficient loader ready.\r\n");

xil_printf("Send 25 coefficients line by line.\r\n");

xil_printf("Accepted input: +0.0011 or EH 0: +0.0011\r\n");

xil_printf("Default kernel after reset: EH 24 = +1.0000, others 0.\r\n\r\n");

while (1) {

/* Poll the UART until a new character arrives. */

while (XUartLite_IsReceiveEmpty(XPAR_XUARTLITE_0_BASEADDR)) {

}

received_byte = XUartLite_RecvByte(XPAR_XUARTLITE_0_BASEADDR);

if (received_byte == '\r' || received_byte == '\n') {

int sign = 1;

int int_part = 0;

int frac_part = 0;

int frac_digits = 0;

int value = 0;

int valid = 1;

int found_number = 0;

char *cursor;

while (XUartLite_IsTransmitFull(XPAR_XUARTLITE_0_BASEADDR)) {

}

XUartLite_SendByte(XPAR_XUARTLITE_0_BASEADDR, '\r');

while (XUartLite_IsTransmitFull(XPAR_XUARTLITE_0_BASEADDR)) {

}

XUartLite_SendByte(XPAR_XUARTLITE_0_BASEADDR, '\n');

input_line[input_length] = '\0';

cursor = input_line;

/* Accept optional leading whitespace and the "EH n:" prefix. */

while (*cursor == ' ' || *cursor == '\t') {

++cursor;

}

if ((cursor[0] == 'E' || cursor[0] == 'e') &&

(cursor[1] == 'H' || cursor[1] == 'h')) {

cursor += 2;

while (*cursor == ' ' || *cursor == '\t') {

++cursor;

}

while (*cursor >= '0' && *cursor <= '9') {

++cursor;

}

while (*cursor == ' ' || *cursor == '\t') {

++cursor;

}

if (*cursor == ':') {

++cursor;

}

}

while (*cursor == ' ' || *cursor == '\t') {

++cursor;

}

if (*cursor == '+') {

++cursor;

} else if (*cursor == '-') {

sign = -1;

++cursor;

}

while (*cursor >= '0' && *cursor <= '9') {

found_number = 1;

int_part = int_part * 10 + (*cursor - '0');

++cursor;

}

if (*cursor == '.') {

++cursor;

while (*cursor >= '0' && *cursor <= '9') {

if (frac_digits < 4) {

frac_part = frac_part * 10 + (*cursor - '0');

}

++frac_digits;

++cursor;

}

}

while (frac_digits < 4) {

frac_part *= 10;

++frac_digits;

}

while (*cursor == ' ' || *cursor == '\t') {

++cursor;

}

if (*cursor != '\0') {

valid = 0;

}

if (!found_number) {

valid = 0;

}

/* More than 4 fractional digits would not match the decimal4 format. */

if (frac_digits > 4) {

valid = 0;

}

if (valid) {

/* Store coefficients as signed integers scaled by 10^4. */

value = sign * (int_part * 10000 + frac_part);

decimal4_coefficients[stored_coefficients] = value;

++stored_coefficients;

xil_printf("Stored coefficient %d/25\r\n", stored_coefficients);

if (stored_coefficients == 25) {

xil_printf("\r\nFull coefficient set received.\r\n");

for (coeff_index = 0; coeff_index < 25; ++coeff_index) {

int abs_value = decimal4_coefficients[coeff_index];

char sign_char = '+';

if (abs_value < 0) {

sign_char = '-';

abs_value = -abs_value;

}

xil_printf("EH %d: %c%d.%04d\r\n",

coeff_index,

sign_char,

abs_value / 10000,

abs_value % 10000);

}

/* Convert and write the full 5x5 coefficient set in register order. */

for (coeff_index = 0; coeff_index < 25; ++coeff_index) {

int q8_8_raw = decimal4_to_q8_8_raw(

decimal4_coefficients[coeff_index]);

Xil_Out32(XPAR_FIR_COEFF_BANK_AXI_0_BASEADDR + 4 * coeff_index,

(u32)q8_8_raw);

}

xil_printf("\r\nCoefficients applied to FIR.\r\n");

xil_printf("Ready for next 25 coefficients.\r\n");

stored_coefficients = 0;

}

} else if (input_length != 0) {

xil_printf("Invalid format. Use +0.0011 or EH n: +0.0011\r\n");

}

/* Start collecting the next input line after Enter. */

input_length = 0;

} else if ((received_byte == 8 || received_byte == 127) && input_length > 0) {

--input_length;

/* Echo backspace in a terminal-friendly way. */

while (XUartLite_IsTransmitFull(XPAR_XUARTLITE_0_BASEADDR)) {

}

XUartLite_SendByte(XPAR_XUARTLITE_0_BASEADDR, 8);

while (XUartLite_IsTransmitFull(XPAR_XUARTLITE_0_BASEADDR)) {

}

XUartLite_SendByte(XPAR_XUARTLITE_0_BASEADDR, ' ');

while (XUartLite_IsTransmitFull(XPAR_XUARTLITE_0_BASEADDR)) {

}

XUartLite_SendByte(XPAR_XUARTLITE_0_BASEADDR, 8);

} else if (input_length < (int)sizeof(input_line) - 1) {

input_line[input_length] = (char)received_byte;

++input_length;

/* Echo regular characters so the typed coefficient is visible on UART. */

while (XUartLite_IsTransmitFull(XPAR_XUARTLITE_0_BASEADDR)) {

}

XUartLite_SendByte(XPAR_XUARTLITE_0_BASEADDR, received_byte);

} else {

xil_printf("\r\nLine too long.\r\n");

input_length = 0;

}

}

return 0;

}

Tesztelés

| Bemenet | Elvárt működés | Megfigyelés | Eredmény |
| --- | --- | --- | --- |
| RST gomb megnyomása | Áteresztő szűrő betöltése | Kiterjesztett képernyő megjelenik változatlanul | PASSED |
| Laplace szűrő együtthatóinak megadása helyesen | Éldetektor a képre | FIR szűrőnél megfigyelt Laplace szűrt kép, csak folytonos stream mellett | PASSED |
| Bemeneti formátum: abc | Hibaüzenet, nem fogadja el | Invalid format | PASSED |
| Törtlés gépelsé közben | Beírt karakter törlése | Beírt karakter törlődött | PASSED |
| Kevesebb mint 25 együttható megadása (24) | Végtelen ideig vár az utolsóra | Képben nem történik változás | PASSED |
| Nincs megadva előjel | Pozitívnak kell vegye | Pozitívnak veszi, ha kiolvassuk a regisztert | PASSED |
| Egész rész lehagyása .1234 | Hibaüzenet | Invalid format | PASSED |